# EAST AFO Meeting Notes 24/08/26

# 1. Executive summary

The EAST application is now running in a dedicated Python 3.10.11 virtual environment and has been synchronised with the latest schi3371/EAST GitHub baseline. The GUI was compacted for the laboratory laptop, the live graph window was enlarged, neutral return was configured to use 0.0 ODrive relative turns as the 90-degree target, and a premature motion-timeout defect was corrected. The current automated suite passes 10 tests. Hardware observations confirm that the ODrive reports relative position only; therefore, 0 turns is an operational reference and does not independently prove physical 90-degree alignment.

# 3. Environment and repository

Installed Python 3.10.11 because the machine initially exposed only an unusable Windows Store alias and Python 3.4.3 bundled with IBM SPSS.

Created a project-local .venv and installed the 67 packages pinned in requirements.txt.

Verified package consistency with pip check and application import checks.

local Git remote is https://github.com/schi3371/EAST.git.

Local main tracks origin/main at commit 537276c (Apply EAST light GUI layout).

# 4. GUI changes

## Main application window

Reduced the responsive main-window height by 20%, from a previous 620-700 px range to 496-560 px.

Removed the empty bottom footer that displayed only the software version.

Reduced outer margins, header height, logo size, input spacing, button spacing, and manual-control heights.

Changed the left controls panel to a vertically scrollable frame so all manual controls remain reachable on short displays.

Changed the body grid to an equal 50/50 width split between the control panel and Session Terminal, making input fields and buttons wider.

## Graph window

Doubled the graph window dimensions from 640 x 420 px to 1280 x 840 px.

Doubled its minimum size from 520 x 340 px to 1040 x 680 px.

Retained the graph as a separate movable Qt window.

# 5. Neutral-position decision

The current implementation defines physical neutral operationally as 0.0 ODrive relative turns. This target is stored in tester_config.json as motion.neutral_position_turns and is loaded on connection.

The manual control is labelled Return to 90 deg (0 turns).

A test can start only when the measured relative position is within the configured 0.05-degree tolerance of zero.

After a successful test, the controller automatically returns to 0.0 turns and then becomes idle.

Run metadata records the fixed neutral target and its reference status.

The earlier session-calibration button was removed to avoid overriding the fixed value.

Reset Form remains a non-moving control: it stops motion and clears fields but does not command a neutral return.

# 6. Motion-timeout incident and correction

## Observed incident

Two runs commanded a range of -10 to +10 degrees, speed 5 deg/s, and acceleration 0.25 deg/s².

Both stopped during moving_to_max after exactly 9.0 seconds, at approximately +8.35 degrees.

The ODrive reported zero active errors and the axis was still progressing toward the target.

Example run files: Salim_test1_3_20260824T123655_318671+1000 and Salim_test1_3_20260824T123842_331037+1000.

## Root cause

The former timeout calculation divided distance by requested speed and added a fixed five-second margin. It ignored commanded acceleration. At 0.25 deg/s², a 10-degree move cannot reach the requested 5 deg/s; it follows a triangular profile with an ideal duration of approximately 12.65 seconds. The nine-second deadline was therefore invalid and stopped otherwise normal motion.

## Correction

Implemented triangular and trapezoidal trajectory-duration calculations using distance, speed, and acceleration.

Corrected the first neutral-to-maximum nominal distance from 20 degrees to its actual 10 degrees.

For the reported settings, the corrected timeout is 17.65 seconds for neutral to +10 degrees and 22.89 seconds for a full +10 to -10 degree traverse, both including the five-second margin.

Retained the configured overall timeout ceiling of 30 seconds.

Added regression tests for low-acceleration triangular motion and trapezoidal motion.

# 7. Load-cell observations

The timeout was not caused by the load cell. However, the two affected runs showed post-tare readings of approximately -36 g and -183 g, and the tare offset changed materially between runs. This warrants separate investigation of settling time, mounting/preload, electrical noise, calibration direction, and whether 16 tare samples are sufficient. The configured calibration remains explicitly provisional.

# 8. Verification completed

| Check | Result |
| --- | --- |
| Python runtime | Python 3.10.11 in .venv |
| Dependency consistency | No broken requirements |
| Automated tests | 10 passed |
| Python compilation | Passed |
| Main application import | Passed without opening hardware |
| ODrive read-only check | Idle, zero errors, relative position available |
| Git state | Five modified tracked files; no commit or push |

# 9. Current limitations and safety considerations

Zero turns is a relative encoder coordinate, not sure if it is an independent measurement of a true physical right angle.

Physical 90-degree alignment must be checked after power cycles, encoder-reference changes, coupling movement, or fixture changes.

A permanent absolute neutral requires an absolute encoder, encoder index procedure, repeatable home switch, or mechanical datum sensor.

The 2.055 AFO-degrees-per-ODrive-turn conversion remains provisional and requires comparison with an independent angular reference.

Load-cell calibration, torque geometry, safety limits, and acceptance criteria remain provisional.

The Stop button and Escape key are software controls and do not replace the physical emergency stop.

Formal AFO testing should not proceed until the verification protocol and acceptance limits are approved.

# 10. Decisions requiring supervisor acknowledgement

Accept 0.0 ODrive relative turns as the interim operational definition of 90-degree neutral, subject to a physical pre-run check.

Decide whether an absolute encoder, home switch, encoder index, or mechanical 90-degree datum should be added before formal data collection.

Confirm acceptable test speed and acceleration ranges, particularly whether very low accelerations such as 0.25 deg/s² are intentional.

Approve a load-cell calibration and tare-stability protocol before interpreting stiffness results.

Approve motion-conversion and speed-verification acceptance limits.

# 11. Recommended next actions

| Priority | Action | Owner | Evidence/output |
| --- | --- | --- | --- |
| High | Repeat a guarded dummy-specimen motion test using the corrected timeout | Researcher | Completed CSV/metadata without timeout |
| High | Verify physical zero/90 degrees before and after an ODrive power cycle | Researcher + supervisor | Independent angle readings and encoder values |
| High | Investigate tare drift and repeatability | Researcher | Zero-load series, mean, SD, drift |
| High | Verify 2.055 deg/turn conversion and commanded speed | Researcher | Independent encoder or video analysis |
| Medium | Select permanent neutral-reference hardware | Supervisory team | Documented design decision |
| Medium | Review GUI visually on the target laptop | Researcher | Screenshots and operator feedback |
| Medium | Review, commit, and push the local changes after approval | Repository owner | Reviewed Git commit/PR |

# 12. Suggested supervisor update

The EAST software environment and latest GUI baseline are operational. I have improved the laptop layout, defined the interim neutral target as zero relative turns, and corrected a timeout that ignored low acceleration. The latest automated suite passes 10 tests. Hardware logs show no ODrive errors during the timeout event, but absolute 90-degree verification and load-cell tare stability remain unresolved experimental requirements. I am seeking approval for the proposed physical verification and calibration work before formal testing.